MIBs for switches

(Note: In the document, if there is an index after the Node name, it needs to be added to the OID and used together during operation;

.1 of ifIndex .1 , the actual operation OID is .1.3.6.1.2.1.2.2.1.1.1 )

Table of contents

MIBs for switches	1

1	Public Node Support	3

1.1	System Message	3

1.2	Port Information	5

1.3	VLAN Information	9

1.4	MAC Address Table	10

1.5	LLDP Configuration Information	11

1.6	LLDP Neighbor Information	12

1.7	STP Information	14

1.8	PoE Information	17

1.9	802.1X Information	19

1.10	IP Information	22

1.11	ARP Information	24

1.12	Route Information	25

1.13	TCP Information	26

1.14	UDP Information	29

2	Private Node Support	31

2.1	System Information	31

2.2	System Operation	31

2.3	IOCTL	32

2.4	PoE	32

2.5	Loop Detect	34

2.6	Power Information	35

2.7	Cable Detect	36

2.8	Port Control	37

2.9	Optical Information	40

3	Trap Notice	44

  - Public Node Support

System Message

(SNMPv2-MIB.iso.org.dod.internet.mgmt.mib-2.system)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| sysDescr | .1.3.6.1.2.1.1.1.0 | system descriptor | OCTET STRING | read-only Implement consistency with mib file definition. |
| sysObjectID | .1.3.6.1.2.1.1.2.0 | Vendor OID | OBJECT IDENTIFIER | read-only Implement consistency with mib file definition. |
| sysUpTime | .1.3.6.1.2.1.1.3.0 | system startup time | TimeTicks | read-only Implement consistency with mib file definition. |
| sysContact | .1.3.6.1.2.1.1.4.0 | technical contact | OCTET STRING | read-write Implement consistency with mib file definition. |
| sysName | .1.3.6.1.2.1.1.5.0 | system name | OCTET STRING | read-write Implement consistency with mib file definition. |
| sysLocation | .1.3.6.1.2.1.1.6.0 | System location information | OCTET STRING | read-write Implement consistency with mib file definition. |

(ENTITY-MIB.iso.org.dod.internet.mgmt.mib-2.entityMIB.entityMIBObjects.entityPhysical)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| entPhysicalIndex.1 | .1.3.6.1.2.1.47.1.1.1.1.1.1 | hardware entity index | Integer32 | not-accessible Implement consistency with mib file definition. |
| entPhysicalDescr.1 | .1.3.6.1.2.1.47.1.1.1.1.2.1 | hardware entity descriptor | OCTET STRING | read-only Implement consistency with mib file definition. |
| entPhysicalVendorType.1 | .1.3.6.1.2.1.47.1.1.1.1.3.1 | Vendor OID | OBJECT IDENTIFIER | read-only Implement consistency with mib file definition. |
| entPhysicalContainedIn.1 | .1.3.6.1.2.1.47.1.1.1.1.4.1 | belongs to which entity | Integer32 | read-only Implement consistency with mib file definition. |
| entPhysicalClass.1 | .1.3.6.1.2.1.47.1.1.1.1.5.1 | hardware type | INTEGER | read-only Implement consistency with mib file definition. |
| entPhysicalParentRelPos.1 | .1.3.6.1.2.1.47.1.1.1.1.6.1 | previous entity number | Integer32 | read-only Implement consistency with mib file definition. |
| entPhysicalName.1 | .1.3.6.1.2.1.47.1.1.1.1.7.1 | entity name | OCTET STRING | read-only Implement consistency with mib file definition. |
| entPhysicalHardwareRev.1 | .1.3.6.1.2.1.47.1.1.1.1.8.1 | hardware version number | OCTET STRING | read-only Implement consistency with mib file definition. |
| entPhysicalFirmwareRev.1 | .1.3.6.1.2.1.47.1.1.1.1.9.1 | Firmware version number | OCTET STRING | read-only Implement consistency with mib file definition. |
| entPhysicalSoftwareRev.1 | .1.3.6.1.2.1.47.1.1.1.1.10.1 | software version number | OCTET STRING | read-only Implement consistency with mib file definition. |
| entPhysicalSerialNum.1 | .1.3.6.1.2.1.47.1.1.1.1.11.1 | serial number | OCTET STRING | read-write Only read operations are supported. |
| entPhysicalMfgName.1 | .1.3.6.1.2.1.47.1.1.1.1.12.1 | Trade Names | OCTET STRING | read-only Implement consistency with mib file definition. |
| entPhysicalModelName.1 | .1.3.6.1.2.1.47.1.1.1.1.13.1 | Device model | OCTET STRING | read-only Implement consistency with mib file definition. |
| entPhysicalAlias.1 | .1.3.6.1.2.1.47.1.1.1.1.14.1 | entity alias | OCTET STRING | read-write Only read operations are supported |
| entPhysicalAssetID.1 | .1.3.6.1.2.1.47.1.1.1.1.15.1 | Entity ID | OCTET STRING | read-write Only read operations are supported. |
| entPhysicalIsFRU.1 | .1.3.6.1.2.1.47.1.1.1.1.16.1 | Whether to support FRU | INTEGER | read-only Implement consistency with mib file definition. |
| entPhysicalMfgDate.1 | .1.3.6.1.2.1.47.1.1.1.1.17.1 | Production Date | OCTET STRING | read-only Implement consistency with mib file definition. |
| entPhysicalUris.1 | .1.3.6.1.2.1.47.1.1.1.1.18.1 | resource descriptor | OCTET STRING | read-write Only read operations are supported. |

Port Information

(IF-MIB.iso.org.dod.internet.mgmt.mib -2.interfaces.ifTable.ifEntry)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| ifNumber | .1.3.6.1.2.1.2.1.0 | Number of ports | Integer32 | read-only Implement consistency with mib file definition. |
| ifIndex.1 | .1.3.6.1.2.1.2.2.1.1 | Index value of interface 1 | Integer32 | read-only Implement consistency with mib file definition. |
| ifDescr.1 | .1.3.6.1.2.1.2.2.1.2 | Descriptor for interface 1 | OCTET STRING | read-only Implement consistency with mib file definition. |
| ifType.1 | .1.3.6.1.2.1.2.2.1.3 | Type of interface 1 | INTEGER | read-only Implement consistency with mib file definition. |
| ifMtu.1 | .1.3.6.1.2.1.2.2.1.4 | MTU of interface 1 | Integer32 | read-only Implement consistency with mib file definition. |
| ifSpeed.1 | .1.3.6.1.2.1.2.2.1.5 | Interface 1 rate | Gauge32 | read-only Implement consistency with mib file definition. |
| ifPhysAddress.1 | .1.3.6.1.2.1.2.2.1.6 | MAC address of interface 1 | OCTET STRING | read-only Implement consistency with mib file definition. |
| ifAdminStatus.1 | .1.3.6.1.2.1.2.2.1.7 | Management state of interface 1 | INTEGER | read-write Implement consistency with mib file definition. |
| ifOperStatus.1 | .1.3.6.1.2.1.2.2.1.8 | Actual state of interface 1 | INTEGER | read-only Implement consistency with mib file definition. |
| ifLastChange.1 | .1.3.6.1.2.1.2.2.1.9 | Time of the last status change of interface 1 | TimeTicks | read-only Implement consistency with mib file definition. |
| ifInOctets.1 | .1.3.6.1.2.1.2.2.1.10 | Number of bytes received by interface 1 | Counter32 | read-only Implement consistency with mib file definition. |
| ifInUcastPkts.1 | .1.3.6.1.2.1.2.2.1.11 | Number of unicast packets received by interface 1 | Counter32 | read-only Implement consistency with mib file definition. |
| ifInNUcastPkts.1 | .1.3.6.1.2.1.2.2.1.12 | Number of non-unicast packets received by interface 1 | Counter32 | read-only Implement consistency with mib file definition. |
| ifInDiscards.1 | .1.3.6.1.2.1.2.2.1.13 | Number of discarded packets received by interface 1 | Counter32 | read-only Implement consistency with mib file definition. |
| ifInErrors.1 | .1.3.6.1.2.1.2.2.1.14 | Number of error packets received by interface 1 | Counter32 | read-only Implement consistency with mib file definition. |
| ifInUnknownProtos.1 | .1.3.6.1.2.1.2.2.1.15 | Number of unknown packets received by interface 1 | Counter32 | read-only Implement consistency with mib file definition. |
| ifOutOctets.1 | .1.3.6.1.2.1.2.2.1.16 | Number of bytes sent by interface 1 | Counter32 | read-only Implement consistency with mib file definition. |
| ifOutUcastPkts.1 | .1.3.6.1.2.1.2.2.1.17 | Number of unicast packets sent by interface 1 | Counter32 | read-only Implement consistency with mib file definition. |
| ifOutNUcastPkts.1 | .1.3.6.1.2.1.2.2.1.18 | Number of non-unicast packets sent by interface 1 | Counter32 | read-only Implement consistency with mib file definition. |
| ifOutDiscards.1 | .1.3.6.1.2.1.2.2.1.19 | Number of lost packets sent by interface 1 | Counter32 | read-only Implement consistency with mib file definition. |
| ifOutErrors.1 | .1.3.6.1.2.1.2.2.1.20 | error packets sent by interface 1 | Counter32 | read-only Implement consistency with mib file definition. |

(IF-MIB.iso.org.dod.internet.mgmt.mib -2.ifMIB.ifMIBObjects.ifXTable.ifXEntry)

Support 64-bit statistics to avoid data crossing or flipping; Note: SNMP V2 C version only supports 64-bit data .

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| ifName.1 | .1.3.6.1.2.1.31.1.1.1.1.1 | interface name | OCTET STRING | read-only Implement consistency with mib file definition. |
| ifInMulticastPkts.1 | .1.3.6.1.2.1.31.1.1.1.2.1 | Number of received multicast packets | Counter32 | read-only Implement consistency with mib file definition. |
| ifInBroadcastPkts.1 | .1.3.6.1.2.1.31.1.1.1.3.1 | The number of received broadcast packets | Counter32 | read-only Implement consistency with mib file definition. |
| ifOutMulticastPkts.1 | .1.3.6.1.2.1.31.1.1.1.4.1 | Number of multicast packets sent | Counter32 | read-only Implement consistency with mib file definition. |
| ifOutBroadcastPkts.1 | .1.3.6.1.2.1.31.1.1.1.5.1 | Number of broadcast packets sent | Counter32 | read-only Implement consistency with mib file definition. |
| ifHCInOctets.1 | .1.3.6.1.2.1.31.1.1.1.6.1 | received bytes | Counter64 | read-only Implement consistency with mib file definition. |
| ifHCInUcastPkts.1 | .1.3.6.1.2.1.31.1.1.1.7.1 | received unicast packets | Counter64 | read-only Implement consistency with mib file definition. |
| ifHCInMulticastPkts.1 | .1.3.6.1.2.1.31.1.1.1.8.1 | Number of received multicast packets | Counter64 | read-only Implement consistency with mib file definition. |
| ifHCInBroadcastPkts.1 | .1.3.6.1.2.1.31.1.1.1.9.1 | The number of received broadcast packets | Counter64 | read-only Implement consistency with mib file definition. |
| ifHCOutOctets.1 | .1.3.6.1.2.1.31.1.1.1.10.1 | send bytes | Counter64 | read-only Implement consistency with mib file definition. |
| ifHCOutUcastPkts.1 | .1.3.6.1.2.1.31.1.1.1.11.1 | Number of unicast packets sent | Counter64 | read-only Implement consistency with mib file definition. |
| ifHCOutMulticastPkts.1 | .1.3.6.1.2.1.31.1.1.1.12.1 | Number of multicast packets sent | Counter64 | read-only Implement consistency with mib file definition. |
| ifHCOutBroadcastPkts.1 | .1.3.6.1.2.1.31.1.1.1.13.1 | Number of broadcast packets sent | Counter64 | read-only Implement consistency with mib file definition. |
| ifLinkUpDownTrapEnable.1 | .1.3.6.1.2.1.31.1.1.1.14.1 | Link notification enable | INTEGER | read-write Implement consistency with mib file definition. |
| ifHighSpeed.1 | .1.3.6.1.2.1.31.1.1.1.15.1 | port speed | Gauge32 | read-only Implement consistency with mib file definition. |
| ifPromiscuousMode.1 | .1.3.6.1.2.1.31.1.1.1.16.1 | promiscuous mode | INTEGER | read-write Only read operations are supported. |
| ifConnectorPresent.1 | .1.3.6.1.2.1.31.1.1.1.17.1 | Is it connected | INTEGER | read-only Implement consistency with mib file definition. |
| ifAlias.1 | .1.3.6.1.2.1.31.1.1.1.18.1 | alias | OCTET STRING | read-write Implement consistency with mib file definition. |

VLAN Information

(Q-BRIDGE - MIB.iso.org.dod.internet.mgmt.mib -2.dot1dBridge.qBridgeMIB.qBridgeMIBObjects.dot1qVlan.dot1qVlanStaticTable.dot1qVlanStaticEntry)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| dot1qVlanStaticName.1 | .1.3.6.1.2.1.17.7.1.4.3.1.1 | Name of VLAN 1 | OCTET STRING | read-create Read and write operations are supported. |
| dot1qVlanStaticEgressPorts.1 | .1.3.6.1.2.1.17.7.1.4.3.1.2 | Quasi-outbound port of VLAN 1 | OCTET STRING | read-create Implement consistency with mib file definition. |
| dot1qVlanForbiddenEgressPorts.1 | .1.3.6.1.2.1.17.7.1.4.3.1.3 | Prohibited outgoing ports of VLAN 1 | OCTET STRING | read-create Implement consistency with mib file definition. |
| dot1qVlanStaticUntaggedPorts.1 | .1.3.6.1.2.1.17.7.1.4.3.1.4 | Untagged port of VLAN 1 | OCTET STRING | read-create Implement consistency with mib file definition. |
| dot1qVlanStaticRowStatus.1 | .1.3.6.1.2.1.17.7.1.4.3.1.5 | Node status for VLAN 1 | INTEGER | read-create Implement consistency with mib file definition. |

MAC Address Table

(Name dot 1qTpFdbEntry!@#.iso.org.dod.internet.mgmt.mib-2.dot1dBridge.qBridgeMIB.qBridgeMIBObjects.dot1qTp.dot1qTpFdbTable.dot1qTpFdbEntry)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| dot1qTpFdbAddress | .1.3.6.1.2.1.17.7.1.2.2.1.1 | Table index, the content is VLAN ID+MAC address | OCTET STRING | not-accessible Implement consistency with mib file definition. |
| dot1qTpFdbPort | .1.3.6.1.2.1.17.7.1.2.2.1.2 | Index of the interface where the MAC address resides | INTEGER | read-only Implement consistency with mib file definition. |
| dot1qTpFdbStatus | .1.3.6.1.2.1.17.7.1.2.2.1.3 | MAC address type | INTEGER | read-only Implement consistency with mib file definition. |

LLDP Configuration Information

(LLDP-V2-MIB-200906080000Z.iso.org.ieee.standards-association-numbers-series-standards.lan-man-stds.ieee802dot1.ieee802dot1-1.lldpV2MIB.lldpV2Objects.lldpV2Configuration.lldpV2PortConfigTable.lldpConfigV2Port)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| lldpV2PortConfigIfIndex | 1.3.111.2.802.1.1.13.1.1.8.1.1 | port index | Integer32 | not-accessible Implement consistency with mib file definition. |
| lldpV2PortConfigDestAddressIndex | 1.3.111.2.802.1.1.13.1.1.8.1.2 | Port destination mac index value | Unsigned32 | not-accessible Implement consistency with mib file definition. |
| lldpV2PortConfigAdminStatus | 1.3.111.2.802.1.1.13.1.1.8.1.3 | management status | INTEGER | read-write Implement consistency with mib file definition. |
| lldpV2PortConfigNotificationEnable | 1.3.111.2.802.1.1.13.1.1.8.1.4 | notification enable | INTEGER | read-write Not currently supported. |
| lldpV2PortConfigTLVsTxEnable | 1.3.111.2.802.1.1.13.1.1.8.1.5 | TLV sending type | BITS | read-write Only supports is set to 0x00. |

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| lldpV2AddressTableIndex | .1.3.111.2.802.1.1.13.1.1.9.1.1 | table index | Unsigned32 | not-accessible Implement consistency with mib file definition. |
| lldpV2DestMacAddress | .1.3.111.2.802.1.1.13.1.1.9.1.2 | MAC address. | OCTET STRING | read-only Implement consistency with mib file definition. |

LLDP Neighbor Information

(LLDP-V2-MIB-200906080000Z.iso.org.ieee.standards-association-numbers-series-standards.lan-man-stds.ieee802dot1.ieee802dot1-1.lldpV2MIB.lldpV2Objects.lldpV2LocalSystemData.lldpV2EntryPortTable.lldpV2Locorg .ieee.standards-association-numbers-series-standards.lan-man-stds.ieee802dot1.ieee802dot1-1.lldpV2MIB.lldpV2Objects.lldpV2RemoteSystemsData.lldpV2RemTable.lldpV2RemEntry)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| lldpV2LocPortIfIndex | 1.3.111.2.802.1.1.13.1.3.7.1.1 | port index | Integer32 | not-accessible Implement consistency with mib file definition. |
| lldpV2LocPortIdSubtype | 1.3.111.2.802.1.1.13.1.3.7.1.2 | port id type | INTEGER | read-only Implement consistency with mib file definition. |
| lldpV2LocPortId | 1.3.111.2.802.1.1.13.1.3.7.1.3 | port id | OCTET STRING | read-only Implement consistency with mib file definition. |
| lldpV2LocPortDesc | 1.3.111.2.802.1.1.13.1.3.7.1.4 | port description | OCTET STRING | read-only Implement consistency with mib file definition. |

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| lldpV2RemTimeMark | 1.3.111.2.802.1.1.13.1.4.1.1.1 | local port index | TimeTicks | not-accessible Implement consistency with mib file definition. |
| lldpV2RemLocalIfIndex | 1.3.111.2.802.1.1.13.1.4.1.1.2 | local port index | Integer32 | not-accessible Implement consistency with mib file definition. |
| lldpV2RemLocalDestMACAddress | 1.3.111.2.802.1.1.13.1.4.1.1.3 | Local destination MAC type | Unsigned32 | not-accessible Implement consistency with mib file definition. |
| lldpV2RemIndex | 1.3.111.2.802.1.1.13.1.4.1.1.4 _ | neighbor index | Unsigned32 | not-accessible Implement consistency with mib file definition. |
| lldpV2RemChassisIdSubtype | 1.3.111.2.802.1.1.13.1.4.1.1.5 | Neighbor Chassis Type | INTEGER | read-only Implement consistency with mib file definition. |
| lldpV2RemChassisId | 1.3.111.2.802.1.1.13.1.4.1.1.6 | Neighbor Chassis Information | OCTET STRING | read-only Implement consistency with mib file definition. |
| lldpV2RemPortIdSubtype | 1.3.111.2.802.1.1.13.1.4.1.1.7 | Neighbor port ID type | INTEGER | read-only Implement consistency with mib file definition. |
| lldpV2RemPortId | 1.3.111.2.802.1.1.13.1.4.1.1.8 | neighbor port ID | OCTET STRING | read-only Implement consistency with mib file definition. |
| lldpV2RemPortDesc | 1.3.111.2.802.1.1.13.1.4.1.1.9 | Neighbor port description | OCTET STRING | read-only Implement consistency with mib file definition. |
| lldpV2RemSysName | 1.3.111.2.802.1.1.13.1.4.1.1.10 | neighbor system name | OCTET STRING | read-only Implement consistency with mib file definition. |
| lldpV2RemSysDesc | 1.3.111.2.802.1.1.13.1.4.1.1.11 | Neighborhood System Description | OCTET STRING | read-only Implement consistency with mib file definition. |
| lldpV2RemSysCapSupported | 1.3.111.2.802.1.1.13.1.4.1.1.12 | Neighboring System Capability Range | BITS | read-only Implement consistency with mib file definition. |
| lldpV2RemSysCapEnabled | 1.3.111.2.802.1.1.13.1.4.1.1.13 | Enabling neighbor system capabilities | BITS | read-only Implement consistency with mib file definition. |
| lldpV2RemRemoteChanges | 1.3.111.2.802.1.1.13.1.4.1.1.14 | Neighborhood Change ID | INTEGER | read-only Implement consistency with mib file definition. |
| lldpV2RemTooManyNeighbors | 1.3.111.2.802.1.1.13.1.4.1.1.15 | Too many neighbors flag | INTEGER | read-only Implement consistency with mib file definition. |

STP Information

(RSTP-MIB iso.org.dod.internet.mgmt.mib-2.dot1dBridge.dot1dStp)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| dot1dStpProtocolSpecification.0 | 1.3.6.1.2.1.17.2.1 |  | INTEGER | read-only Read result is fixed ieee8021d(3). |
| dot1dStpPriority.0 | 1.3.6.1.2.1.17.2.2 |  | INTEGER | read-write Implement consistency with mib file definition. |
| dot1dStpTimeSinceTopologyChange.0 | 1.3.6.1.2.1.17.2.3 |  | TimeTicks | read-only Implement consistency with mib file definition. |
| dot1dStpTopChanges.0 | 1.3.6.1.2.1.17.2.4 |  | Counter | read-only Implement consistency with mib file definition. |
| dot1dStpDesignatedRoot.0 | 1.3.6.1.2.1.17.2.5 |  | OCTET STRING | read-only Implement consistency with mib file definition. |
| dot1dStpRootPort.0 | 1.3.6.1.2.1.17.2.6 |  | INTEGER | read-only Implement consistency with mib file definition. |
| dot1dStpRootPort.0 | 1.3.6.1.2.1.17.2.7 |  | INTEGER | read-only Implement consistency with mib file definition. |
| dot1dStpMaxAge.0 | 1.3.6.1.2.1.17.2.8 |  | INTEGER | read-only Implement consistency with mib file definition. |
| dot1dStpHelloTime.0 | 1.3.6.1.2.1.17.2.9 |  | INTEGER | read-only Implement consistency with mib file definition. |
| dot1dStpHoldTime.0 | 1.3.6.1.2.1.17.2.10 |  | INTEGER | read-only Implement consistency with mib file definition. |
| dot1dStpForwardDelay.0 | 1.3.6.1.2.1.17.2.11 |  | INTEGER | read-only Implement consistency with mib file definition. |
| dot1dStpBridgeMaxAge.0 | 1.3.6.1.2.1.17.2.12 |  | INTEGER | read-write Implement consistency with mib file definition. |
| dot1dStpBridgeHelloTime.0 | 1.3.6.1.2.1.17.2.13 |  | INTEGER | read-write Implement consistency with mib file definition. |
| dot1dStpBridgeForwardDelay.0 | 1.3.6.1.2.1.17.2.14 |  | INTEGER | read-write Implement consistency with mib file definition. |
| dot1dStpPort.1 | 1.3.6.1.2.1.17.2.15.1.1 |  | INTEGER | read-only Implement consistency with mib file definition. |
| dot1dStpPortPriority.1 | 1.3.6.1.2.1.17.2.15.1.2 |  | INTEGER | read-write Implement consistency with mib file definition. |
| dot1dStpPortState.1 | 1.3.6.1.2.1.17.2.15.1.3 |  | INTEGER | read-only Implement consistency with mib file definition. |
| dot1dStpPortEnable.1 | 1.3.6.1.2.1.17.2.15.1.4 |  | INTEGER | read-write Implement consistency with mib file definition. |
| dot1dStpPortPathCost.1 | 1.3.6.1.2.1.17.2.15.1.5 |  | INTEGER | read-write Implement consistency with mib file definition. |
| dot1dStpPortDesignatedRoot.1 | 1.3.6.1.2.1.17.2.15.1.6 |  | OCTET STRING | read-only Implement consistency with mib file definition. |
| dot1dStpPortDesignatedCost.1 | 1.3.6.1.2.1.17.2.15.1.7 |  | INTEGER | read-only Implement consistency with mib file definition. |
| dot1dStpPortDesignatedBridge.1 | 1.3.6.1.2.1.17.2.15.1.8 |  | OCTET STRING | read-only Implement consistency with mib file definition. |
| dot1dStpPortDesignatedPort.1 | 1.3.6.1.2.1.17.2.15.1.9 |  | OCTET STRING | read-only Implement consistency with mib file definition. |
| dot1dStpPortForwardTransitions.1 | 1.3.6.1.2.1.17.2.15.1.10 |  | Counter | read-only Implement consistency with mib file definition. |
| dot1dStpPortPathCost32.1 | 1.3.6.1.2.1.17.2.15.1.11 |  | INTEGER | read-only Implement consistency with mib file definition. |
| dot1dStpVersion.0 | 1.3.6.1.2.1.17.2.16 |  | INTEGER | read-write Implement consistency with mib file definition. |
| dot1dStpTxHoldCount.0 | 1.3.6.1.2.1.17.2.17 |  | INTEGER | read-write Implement consistency with mib file definition. |
| dot1dStpPortProtocolMigration.1 | 1.3.6.1.2.1.17.2.19.1.1 |  | INTEGER | read-write Implement consistency with mib file definition. |
| dot1dStpPortAdminEdgePort.1 | 1.3.6.1.2.1.17.2.19.1.2 |  | INTEGER | read-write Implement consistency with mib file definition. |
| dot1dStpPortOperEdgePort.1 | 1.3.6.1.2.1.17.2.19.1.3 |  | INTEGER | read-only Implement consistency with mib file definition. |
| dot1dStpPortAdminPointToPoint.1 | 1.3.6.1.2.1.17.2.19.1.4 |  | INTEGER | read-write Implement consistency with mib file definition. |
| dot1dStpPortOperPointToPoint.1 | 1.3.6.1.2.1.17.2.19.1.5 |  | INTEGER | read-only Implement consistency with mib file definition. |
| dot1dStpPortAdminPathCost.1 | 1.3.6.1.2.1.17.2.19.1.6 |  | INTEGER | read-write Implement consistency with mib file definition. |

PoE Information

(POWER-ETHERNET-MIB.iso.org.dod.internet.mgmt.mib-2.powerEthernetMIB.pethObjects)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| pethPsePortGroupIndex.1.1 | .1.3.6.1.2.1.105.1.1.1.1.1.1 | group index | Integer32 | not-accessible Implement consistency with mib file definition. |
| pethPsePortIndex.1.1 | .1.3.6.1.2.1.105.1.1.1.2.1.1 | port index | Integer32 | not-accessible Implement consistency with mib file definition. |
| pethPsePortAdminEnable.1.1 | .1.3.6.1.2.1.105.1.1.1.3.1.1 | Management Enabled Status | INTEGER | read-write Implement consistency with mib file definition. |
| pethPsePortPowerPairsControlAbility.1.1 | .1.3.6.1.2.1.105.1.1.1.4.1.1 | Power supply capacity | INTEGER | read-only Implement consistency with mib file definition. |
| pethPsePortPowerPairs.1.1 | .1.3.6.1.2.1.105.1.1.1.5.1.1 | Power supply | INTEGER | read-write Only read operations are supported. |
| pethPsePortDetectionStatus.1.1 | .1.3.6.1.2.1.105.1.1.1.6.1.1 | Port Power Status | INTEGER | read-only Implement consistency with mib file definition. |
| pethPsePortPowerPriority.1.1 | .1.3.6.1.2.1.105.1.1.1.7.1.1 | Power priority | INTEGER | read-only Implement consistency with mib file definition. |
| pethPsePortMPSAbsentCounter.1.1 | .1.3.6.1.2.1.105.1.1.1.8.1.1 | off statistics | Counter32 | read-only Implement consistency with mib file definition. |
| pethPsePortType.1.1 | .1.3.6.1.2.1.105.1.1.1.9.1.1 | port descriptor | OCTET STRING | read-write Implement consistency with mib file definition. |
| pethPsePortPowerClassifications.1.1 | .1.3.6.1.2.1.105.1.1.1.10.1.1 | Power supply type | INTEGER | read-only Implement consistency with mib file definition. |
| pethPsePortInvalidSignatureCounter.1.1 | .1.3.6.1.2.1.105.1.1.1.11.1.1 | Illegal Signal Statistics | Counter32 | read-only Implement consistency with mib file definition. |
| pethPsePortPowerDeniedCounter.1.1 | .1.3.6.1.2.1.105.1.1.1.12.1.1 | Denial of Power Statistics | Counter32 | read-only Implement consistency with mib file definition. |
| pethPsePortOverLoadCounter r.1.1 | .1.3.6.1.2.1.105.1.1.1.13.1.1 | Overload Power Down Statistics | Counter32 | read-only Implement consistency with mib file definition. |
| pethPsePortShortCounter.1.1 | .1.3.6.1.2.1.105.1.1.1.14.1.1 | Port short circuit statistics | Counter32 | read-only Implement consistency with mib file definition. |

802.1X Information

(IEEE8021-PAE- MIB.iso.std.iso8802.ieee802dot1.ieee802dot1mibs.ieee8021paeMIB)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| dot1xPaeSystemAuthControl.0 | .1.0.8802.1.1.1.1.1.1.0 | global authentication switch | INTEGER | read-write Implement consistency with mib file definition. |

(dot1 xPaePortEntry!@#.iso.std.iso8802.ieee802dot1.ieee802dot1mibs.ieee8021paeMIB.paeMIBObjects.dot1xPaeSystem.dot1xPaePortTable.dot1xPaePortEntry )

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| dot1xPaePortNumber.5 | .1.0.8802.1.1.1.1.1.2.1.1.5 | port number | Integer32 | not-accessible Implement consistency with mib file definition. |
| dot1xPaePortProtocolVersion.5 | .1.0.8802.1.1.1.1.1.2.1.2.5 | port protocol version | Unsigned32 | read-only Implement consistency with mib file definition. |
| dot1xPaePortCapabilities.5 | .1.0.8802.1.1.1.1.1.2.1.3.5 | port capability | BITS | read-only Implement consistency with mib file definition. |
| dot1xPaePortInitialize.5 | .1.0.8802.1.1.1.1.1.2.1.4.5 | port initialization flag | INTEGER | read-write Implement consistency with mib file definition. |
| dot1xPaePortReauthenticate.5 | .1.0.8802.1.1.1.1.1.2.1.5.5 | Port re-authentication identifier | INTEGER | read-write Implement consistency with mib file definition. |

( dot1 xAuthConfigEntry!@#.iso.std.iso8802.ieee802dot1.ieee802dot1mibs.ieee8021paeMIB.paeMIBObjects.dot1xPaeAuthenticator.dot1xAuthConfigTable.dot1xAuthConfigEntry )

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| dot1xAuthPaeState.5 | .1.0.8802.1.1.1.1.2.1.1.1.5 | PAE certification status | INTEGER | read-only Implement consistency with mib file definition. |
| dot1xAuthBackendAuthState.5 | .1.0.8802.1.1.1.1.2.1.1.2.5 | BE certification status | INTEGER | read-only Implement consistency with mib file definition. |
| dot1xAuthAdminControlledDirections.5 | .1.0.8802.1.1.1.1.2.1.1.3.5 | Port Controlled Management Direction | INTEGER | read-write Implement consistency with mib file definition. |
| dot1xAuthOperControlledDirections.5 | .1.0.8802.1.1.1.1.2.1.1.4.5 | Port controlled current direction | INTEGER | read-only Implement consistency with mib file definition. |
| dot1xAuthAuthControlledPortStatus.5 | .1.0.8802.1.1.1.1.2.1.1.5.5 | Port controlled state | INTEGER | read-only Implement consistency with mib file definition. |
| dot1xAuthAuthControlledPortControl.5 | .1.0.8802.1.1.1.1.2.1.1.6.5 | port controlled mode | INTEGER | read-write Implement consistency with mib file definition. |
| dot1xAuthQuietPeriod.5 | .1.0.8802.1.1.1.1.2.1.1.7.5 | Port authentication silent period | Unsigned32 | read-write Only read operations are supported. |
| dot1xAuthTxPeriod.5 | .1.0.8802.1.1.1.1.2.1.1.8.5 | sending cycle | Unsigned32 | read-write Implement consistency with mib file definition. |
| dot1xAuthSuppTimeout.5 | .1.0.8802.1.1.1.1.2.1.1.9.5 | client timeout | Unsigned32 | read-write Implement consistency with mib file definition. |
| dot1xAuthServerTimeout.5 | .1.0.8802.1.1.1.1.2.1.1.10.5 | server timeout | Unsigned32 | read-write Implement consistency with mib file definition. |
| dot1xAuthReAuthPeriod.5 | .1.0.8802.1.1.1.1.2.1.1.12.5 | re-authentication cycle | Unsigned32 | read-write Implement consistency with mib file definition. |
| dot1xAuthReAuthEnabled.5 | .1.0.8802.1.1.1.1.2.1.1.13.5 | Enable re-authentication | INTEGER | read-write Implement consistency with mib file definition. |
| dot1xAuthKeyTxEnabled.5 | .1.0.8802.1.1.1.1.2.1.1.14.5 | key send enable | INTEGER | read-write Implement consistency with mib file definition. |

(dot1 xAuthStatsEntry!@#.iso.std.iso8802.ieee802dot1.ieee802dot1mibs.ieee8021paeMIB.paeMIBObjects.dot1xPaeAuthenticator.dot1xAuthStatsTable.dot1xAuthStatsEntry )

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| dot1xAuthEapolFramesRx.5 | .1.0.8802.1.1.1.1.2.2.1.1.5 | Eapol received packets | Counter32 | read-only Implement consistency with mib file definition. |
| dot1xAuthEapolFramesTx.5 | .1.0.8802.1.1.1.1.2.2.1.2.5 | The number of packets sent by Eapol | Counter32 | read-only Implement consistency with mib file definition. |
| dot1xAuthEapolStartFramesRx.5 | .1.0.8802.1.1.1.1.2.2.1.3.5 | The number of packets received at the beginning of Eapol | Counter32 | read-only Implement consistency with mib file definition. |
| dot1xAuthEapolLogoffFramesRx.5 | .1.0.8802.1.1.1.1.2.2.1.4.5 | Eapol offline packets received | Counter32 | read-only Implement consistency with mib file definition. |
| dot1xAuthEapolRespIdFramesRx.5 | .1.0.8802.1.1.1.1.2.2.1.5.5 | Eapol response identification packets | Counter32 | read-only Implement consistency with mib file definition. |
| dot1xAuthEapolRespFramesRx.5 | .1.0.8802.1.1.1.1.2.2.1.6.5 | Eapol response packets received | Counter32 | read-only Implement consistency with mib file definition. |
| dot1xAuthEapolReqIdFramesTx.5 | .1.0.8802.1.1.1.1.2.2.1.7.5 | Eapol request identification packets sent | Counter32 | read-only Implement consistency with mib file definition. |
| dot1xAuthEapolReqFramesTx.5 | .1.0.8802.1.1.1.1.2.2.1.8.5 | Eapol request packet number | Counter32 | read-only Implement consistency with mib file definition. |
| dot1xAuthInvalidEapolFramesRx.5 | .1.0.8802.1.1.1.1.2.2.1.9.5 | illegal Eapol packets received | Counter32 | read-only Implement consistency with mib file definition. |
| dot1xAuthEapLengthErrorFramesRx.5 | .1.0.8802.1.1.1.1.2.2.1.10.5 | Eapol length error packets | Counter32 | read-only Implement consistency with mib file definition. |
| dot1xAuthLastEapolFrameVersion.5 | .1.0.8802.1.1.1.1.2.2.1.11.5 | The latest Eapol message version | Unsigned32 | read-only Implement consistency with mib file definition. |
| dot1xAuthLastEapolFrameSource.5 | .1.0.8802.1.1.1.1.2.2.1.12.5 | latest Eapol message | OCTET STRING | read-only Implement consistency with mib file definition. |

IP Information

(IP-MIB.iso.org.dod.internet.mgmt.mib - 2.ip.ipAddressTable.ipAddressEntrys)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| ipAddressAddrType.1.192.168.100.96 | .1.3.6.1.2.1.4.34.1.1.1.192.168.100.96 | IP address type | INTEGER | not-accessible Implement consistency with mib file definition. |
| ipAddressAddr.1.192.168.100.96 | .1.3.6.1.2.1.4.34.1.2.1.192.168.100.96 | IP address | OCTET STRING | not-accessible Implement consistency with mib file definition. |
| ipAddressIfIndex.1.192.168.100.96 | .1.3.6.1.2.1.4.34.1.3.1.192.168.100.96 | Interface internal index number | INTEGER | read-create Only read operations are supported. |
| ipAddressType.1.192.168.100.96 | .1.3.6.1.2.1.4.34.1.4.1.192.168.100.96 | address type | INTEGER | read-create Only read operations are supported. |
| ipAddressPrefix.1.192.168.100.96 | .1.3.6.1.2.1.4.34.1.5.1.192.168.100.96 | address mask, not supported here | OID | Not currently supported. |
| ipAddressOrigin.1.192.168.100.96 | .1.3.6.1.2.1.4.34.1.6.1.192.168.100.96 | source of address | INTEGER | read-only Implement consistency with mib file definition. |
| ipAddressStatus.1.192.168.100.96 | .1.3.6.1.2.1.4.34.1.7.1.192.168.100.96 | address status | INTEGER | read-create Only read operations are supported. |
| ipAddressCreated.1.192.168.100.96 | .1.3.6.1.2.1.4.34.1.8.1.192.168.100.96 | creation time | TimeTicks | read-only Implement consistency with mib file definition. |
| ipAddressLastChanged.1.192.168.100.96 | .1.3.6.1.2.1.4.34.1.9.1.192.168.100.96 | last change time | TimeTicks | read-only Implement consistency with mib file definition. |
| ipAddressRowStatus.1.192.168.100.96 | .1.3.6.1.2.1.4.34.1.10.1.192.168.100.96 | SNMP status | INTEGER | read-create Read and write operations are supported, and only support set to destroy (6). |
| ipAddressStorageType.1.192.168.100.96 | .1.3.6.1.2.1.4.34.1.11.1.192.168.100.96 | storage type | INTEGER | read-create Only read operations are supported. |

Note: Since this MIB table does not support address mask configuration, the length of the IP address mask created through the MIB table is always 32. Care should be taken when using this MIB table to create IP addresses.

ARP Information

(IP-MIB.iso.org.dod.internet.mgmt.mib - 2.ip.ipNetToPhysicalTable.ipNetToPhysicalEntry)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| ipNetToPhysicalIfIndex.2100.1.192.168.100.106 | .1.3.6.1.2.1.4.35.1.1.2100.1.192.168.100.106 | Port index | Integer32 | not-accessible Implement consistency with mib file definition. |
| ipNetToPhysicalAddressType.2100.1.192.168.100.106 | .1.3.6.1.2.1.4.35.1.2.2100.1.192.168.100.106 | IP address type | INTEGER | not-accessible Implement consistency with mib file definition. |
| ipNetToPhysicalAddress.2100.1.192.168.100.106 | .1.3.6.1.2.1.4.35.1.3.2100.1.192.168.100.106 | IP address | OCTET STRING | not-accessible Implement consistency with mib file definition. |
| ipNetToPhysicalPhysAddress.2100.1.192.168.100.106 | .1.3.6.1.2.1.4.35.1.4.2100.1.192.168.100.106 | MAC address | OCTET STRING | read-create Only read operations are supported. |
| ipNetToPhysicalLastUpdated.2100.1.192.168.100.106 | .1.3.6.1.2.1.4.35.1.5.2100.1.192.168.100.106 | last update timestamp | TimeTicks | read-only Implement consistency with mib file definition. |
| ipNetToPhysicalType.2100.1.192.168.100.106 | .1.3.6.1.2.1.4.35.1.6.2100.1.192.168.100.106 | ARP type | INTEGER | read-create Only read operations are supported. |
| ipNetToPhysicalState.2100.1.192.168.100.106 | .1.3.6.1.2.1.4.35.1.7.2100.1.192.168.100.106 | ARP status | INTEGER | read-only Implement consistency with mib file definition. |
| ipNetToPhysicalRowStatus.2100.1.192.168.100.106 | .1.3.6.1.2.1.4.35.1.8.2100.1.192.168.100.106 | SNMP status | INTEGER | read-create Only read operations are supported. |

Route Information

( IP- FORWARD - MIB.iso.org.dod.internet.mgmt.mib - 2.ip.ipForward.ipCidrRouteTable.ipCidrRouteEntry )

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| ipCidrRouteDest.2.0.2.0.255.255.255.0.0.1.1.1.4 | .1.3.6.1.2.1.4.24.4.1.1.2.0.2.0.255.255.255.0.0.1.1.1.4 | target address | IpAddress | read-only Implement consistency with mib file definition. |
| ipCidrRouteMask.2.0.2.0.255.255.255.0.0.1.1.1.4 | .1.3.6.1.2.1.4.24.4.1.2.2.0.2.0.255.255.255.0.0.1.1.1.4 | target mask | IpAddress | read-only Implement consistency with mib file definition. |
| ipCidrRouteNextHop.2.0.2.0.255.255.255.0.0.1.1.1.4 | .1.3.6.1.2.1.4.24.4.1.4.2.0.2.0.255.255.255.0.0.1.1.1.4 | next hop address | IpAddress | read-only Implement consistency with mib file definition. |
| ipCidrRouteIfIndex.2.0.2.0.255.255.255.0.0.1.1.1.4 | .1.3.6.1.2.1.4.24.4.1.5.2.0.2.0.255.255.255.0.0.1.1.1.4 | interface index | Integer32 | read-create Only read operations are supported. |
| ipCidrRouteType.2.0.2.0.255.255.255.0.0.1.1.1.4 | .1.3.6.1.2.1.4.24.4.1.6.2.0.2.0.255.255.255.0.0.1.1.1.4 | routing type | INTEGER | read-create Only read operations are supported. |
| ipCidrRouteProto.2.0.2.0.255.255.255.0.0.1.1.1.4 | .1.3.6.1.2.1.4.24.4.1.7.2.0.2.0.255.255.255.0.0.1.1.1.4 | Routing Protocol | INTEGER | read-only Implement consistency with mib file definition. |
| ipCidrRouteAge.2.0.2.0.255.255.255.0.0.1.1.1.4 | .1.3.6.1.2.1.4.24.4.1.8.2.0.2.0.255.255.255.0.0.1.1.1.4 | Timestamp since last update | Integer32 | read-only Implement consistency with mib file definition. |
| ipCidrRouteInfo.2.0.2.0.255.255.255.0.0.1.1.1.4 | .1.3.6.1.2.1.4.24.4.1.9.2.0.2.0.255.255.255.0.0.1.1.1.4 | Routing protocol OID, not supported here | OBJECT IDENTIFIER | Not currently supported. |
| ipCidrRouteMetric1.2.0.2.0.255.255.255.0.0.1.1.1.4 | .1.3.6.1.2.1.4.24.4.1.11.2.0.2.0.255.255.255.0.0.1.1.1.4 | Routing metric value | Integer32 | read-create Only read operations are supported. |
| ipCidrRouteStatus.2.0.2.0.255.255.255.0.0.1.1.1.4 | .1.3.6.1.2.1.4.24.4.1.16.2.0.2.0.255.255.255.0.0.1.1.1.4 | SNMP status | INTEGER | read-create Implement consistency with mib file definition. |

(IPV6-MIB .iso.org.dod.internet.mgmt.mib-2.ipv6MIB.ipv6MIBObjects.ipv6RouteTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| ipv6RouteDest | 1.3.6.1.2.1.55.1.11.1.1 | The IPv6 address for this route | OCTET STRING | not-accessible Implement consistency with mib file definition. |
| ipv6RoutePfxLength | 1.3.6.1.2.1.55.1.11.1.2 | Indicates the prefix length of the destination address | INTEGER | not-accessible Implement consistency with mib file definition. |
| ipv6RouteIndex | 1.3.6.1.2.1.55.1.11.1.3 | Route index | Unsigned32 | not-accessible Implement consistency with mib file definition. |
| ipv6RouteIfIndex | 1.3.6.1.2.1.55.1.11.1.4 | The index of the local interface | Integer32 | read-only Implement consistency with mib file definition. |
| ipv6RouteNextHop | 1.3.6.1.2.1.55.1.11.1.5 | Indicates the IPv6 address of the next hop | OCTET STRING | read-only Implement consistency with mib file definition. |
| ipv6RouteType | 1.3.6.1.2.1.55.1.11.1.6 | Route type | INTEGER | read-only Implement consistency with mib file definition. |
| ipv6RouteProtocol | 1.3.6.1.2.1.55.1.11.1.7 | Routing protocol | INTEGER | read-only Implement consistency with mib file definition. |
| ipv6RoutePolicy | 1.3.6.1.2.1.55.1.11.1.8 | Routing policy | Unsigned32 | read-only Implement consistency with mib file definition. |
| ipv6RouteAge | 1.3.6.1.2.1.55.1.11.1.9 | The number of seconds since the last update | Unsigned32 | read-only Implement consistency with mib file definition. |
| ipv6RouteNextHopRDI | 1.3.6.1.2.1.55.1.11.1.10 | ID of the routing domain of the next hop | Unsigned32 | read-only Implement consistency with mib file definition. |
| ipv6RouteMetric | 1.3.6.1.2.1.55.1.11.1.11 | The metric value of the route | Unsigned32 | read-only Implement consistency with mib file definition. |
| ipv6RouteWeight | 1.3.6.1.2.1.55.1.11.1.12 | The internal system weight of the route | Unsigned32 | read-only Implement consistency with mib file definition. |
| ipv6RouteInfo | 1.3.6.1.2.1.55.1.11.1.13 | Reference to the MIB definition for a particular routing protocol | OID | read-only Implement consistency with mib file definition. |
| ipv6RouteValid | 1.3.6.1.2.1.55.1.11.1.14 | Whether the route entry information is valid | Unsigned32 | read-write Only read operations are supported. |

TCP Information

(TCP-MIB. iso.org.dod.internet.mgmt.mib-2.tcp.tcpConnTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| tcpConnState | 1.3.6.1.2.1.6.13.1.1 | TCP connection status | INTEGER | read-write Only read operations are supported. |
| tcpConnLocalAddress | 1.3.6.1.2.1.6.13.1.2 | Local IP address of the TCP connection | IpAddress | read-only Implement consistency with mib file definition. |
| tcpConnLocalPort | 1.3.6.1.2.1.6.13.1.3 | The local port on which the TCP connection listens | Integer32 | read-only Implement consistency with mib file definition. |
| tcpConnRemAddress | 1.3.6.1.2.1.6.13.1.4 | Remote IP address of the TCP connection | IpAddress | read-only Implement consistency with mib file definition. |
| tcpConnRemPort | 1.3.6.1.2.1.6.13.1.5 | Remote port number of the TCP connection | Integer32 | read-only Implement consistency with mib file definition. |

(IPV6-TCP-MIB. iso.org.dod.internet.mgmt.mib-2.tcp.ipv6TcpConnTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| ipv6TcpConnLocalAddress | 1.3.6.1.2.1.6.16.1.1 | Local IPv6 address of the TCP connection | OCTET STRING | not-accessible Implement consistency with mib file definition. |
| ipv6TcpConnLocalPort | 1.3.6.1.2.1.6.16.1.2 | Local port number of the TCP connection | INTEGER | not-accessible Implement consistency with mib file definition. |
| ipv6TcpConnRemAddress | 1.3.6.1.2.1.6.16.1.3 | IPv6 address of the peer end of the TCP connection | OCTET STRING | not-accessible Implement consistency with mib file definition. |
| ipv6TcpConnRemPort | 1.3.6.1.2.1.6.16.1.4 | Port number of the peer end of the TCP connection | INTEGER | not-accessible Implement consistency with mib file definition. |
| ipv6TcpConnIfIndex | 1.3.6.1.2.1.6.16.1.5 | Interface index of the TCP connection | Integer32 | not-accessible Implement consistency with mib file definition. |
| ipv6TcpConnState | 1.3.6.1.2.1.6.16.1.6 | Status of the TCP connection | INTEGER | read-write Only read operations are supported. |

(TCP-MIB. iso.org.dod.internet.mgmt.mib-2.tcp.tcpConnectionTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| tcpConnectionLocalAddressType | 1.3.6.1.2.1.6.19.1.1 | Local IP address type of the TCP connection | INTEGER | not-accessible Implement consistency with mib file definition. |
| tcpConnectionLocalAddress | 1.3.6.1.2.1.6.19.1.2 | Local IP address of the TCP connection | OCTET STRING | not-accessible Implement consistency with mib file definition. |
| tcpConnectionLocalPort | 1.3.6.1.2.1.6.19.1.3 | The local port on which the TCP connection listens | Unsigned32 | not-accessible Implement consistency with mib file definition. |
| tcpConnectionRemAddressType | 1.3.6.1.2.1.6.19.1.4 | Remote IP address type of the TCP connection | INTEGER | not-accessible Implement consistency with mib file definition. |
| tcpConnectionRemAddress | 1.3.6.1.2.1.6.19.1.5 | Remote IP address of the TCP connection | OCTET STRING | not-accessible Implement consistency with mib file definition. |
| tcpConnectionRemPort | 1.3.6.1.2.1.6.19.1.6 | Remote port number of the TCP connection | Unsigned32 | not-accessible Implement consistency with mib file definition. |
| tcpConnectionState | 1.3.6.1.2.1.6.19.1.7 | TCP connection status | INTEGER | read-write Only read operations are supported. |
| tcpConnectionProcess | 1.3.6.1.2.1.6.19.1.8 | Process the TASK ID of a TCP connection | Unsigned32 | read-only Implement consistency with mib file definition. |

(TCP-MIB. iso.org.dod.internet.mgmt.mib-2.tcp.tcpListenerTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| tcpListenerLocalAddressType | 1.3.6.1.2.1.6.20.1.1 | Local IP address type of the TCP connection | INTEGER | not-accessible Implement consistency with mib file definition. |
| tcpListenerLocalAddress | 1.3.6.1.2.1.6.20.1.2 | Local IP address of the TCP connection | OCTET STRING | not-accessible Implement consistency with mib file definition. |
| tcpListenerLocalPort | 1.3.6.1.2.1.6.20.1.3 | The local port on which the TCP connection listens | Unsigned32 | not-accessible Implement consistency with mib file definition. |
| tcpListenerProcess | 1.3.6.1.2.1.6.20.1.4 | System process number | Unsigned32 | read-only Implement consistency with mib file definition. |

UDP Information

(UDP-MIB. iso.org.dod.internet.mgmt.mib-2.udp.udpTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| udpLocalAddress | 1.3.6.1.2.1.7.5.1.1 | Local IP address of a UDP listening endpoint | IpAddress | read-only Implement consistency with mib file definition. |
| udpLocalPort | 1.3.6.1.2.1.7.5.1.2 | Local port number of the UDP listening endpoint | Integer32 | read-only Implement consistency with mib file definition. |

(IPV6-UDP-MIB. iso.org.dod.internet.mgmt.mib-2.udp.ipv6UdpTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| ipv6UdpLocalAddress | 1.3.6.1.2.1.7.6.1.1 | Local IPv6 address listened by UDP | OCTET STRING | not-accessible Implement consistency with mib file definition. |
| ipv6UdpLocalPort | 1.3.6.1.2.1.7.6.1.2 | Number of the local port monitored by UDP | INTEGER | not-accessible Implement consistency with mib file definition. |
| ipv6UdpIfIndex | 1.3.6.1.2.1.7.6.1.3 | Indicates the index of the UDP listening interface | Integer32 | read-only Implement consistency with mib file definition. |

(UDP-MIB. iso.org.dod.internet.mgmt.mib-2.udp.udpEndpointTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| udpEndpointLocalAddressType | 1.3.6.1.2.1.7.7.1.1 | Type of the local IP address monitored by UDP | INTEGER | not-accessible Implement consistency with mib file definition. |
| udpEndpointLocalAddress | 1.3.6.1.2.1.7.7.1.2 | The local IP address on which UDP listens | OCTET STRING | not-accessible Implement consistency with mib file definition. |
| udpEndpointLocalPort | 1.3.6.1.2.1.7.7.1.3 | Local port number of the UDP node | Unsigned32 | not-accessible Implement consistency with mib file definition. |
| udpEndpointRemoteAddressType | 1.3.6.1.2.1.7.7.1.4 | Type of the remote IP address monitored by UDP | INTEGER | not-accessible Implement consistency with mib file definition. |
| udpEndpointRemoteAddress | 1.3.6.1.2.1.7.7.1.5 | The local address of UDP | OCTET STRING | not-accessible Implement consistency with mib file definition. |
| udpEndpointRemotePort | 1.3.6.1.2.1.7.7.1.6 | Remote port number of the UDP node | Unsigned32 | not-accessible Implement consistency with mib file definition. |
| udpEndpointInstance | 1.3.6.1.2.1.7.7.1.7 | The instance number field of the UDP connection | Unsigned32 | not-accessible Implement consistency with mib file definition. |
| udpEndpointProcess | 1.3.6.1.2.1.7.7.1.8 | System process number | Unsigned32 | read-only Implement consistency with mib file definition. |

Private Node Support

System Information

(NMS-SYS-MIB iso.org.dod.internet.private.enterprises.nms.nmsModule.sys.sysModule)

| Node name | OID | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| cpuUtilized.0 | .1.3.6.1.4.1.89089.1.2.1.1.0 | CPU utilization | Integer32 | read-only Implement consistency with mib file definition. |
| memUtilized.0 | .1.3.6.1.4.1.89089.1.2.1.2.0 | Memory utilization | Integer32 | read-only Implement consistency with mib file definition. |
| cpuIdle.0 | .1.3.6.1.4.1.89089.1.2.1.3.0 | CPU idle ratio | Integer32 | read-only Implement consistency with mib file definition. |
| memTotalReal.0 | .1.3.6.1.4.1.89089.1.2.1.4.0 | Total memory, in KB | Integer32 | read-only Implement consistency with mib file definition. |
| memAvailReal.0 | .1.3.6.1.4.1.89089.1.2.1.5.0 | Available memory, in KB | Integer32 | read-only Implement consistency with mib file definition. |

(NMS-SYS-MIB iso.org.dod.internet.private.enterprises.nms.nmsModule.sys.sysModule)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| temperatureIndex | 1.3.6.1.4.1.89089.1.2.4.1.1 | Temperature sensor index | Integer32 | read-only Implement consistency with mib file definition. |
| temperatureValue | 1.3.6.1.4.1.89089.1.2.4.1.2 | Temperature sensor value | Integer32 | read-only Implement consistency with mib file definition. |

System Operation

(NMS-SYS-MIB.iso.org.dod.internet.private.enterprises.nms.nmsModule.sys.operations)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| configurationOper.0 | .1.3.6.1.4.1.89089.1.2.2.1.0 | Configuration operations | INTEGER { save (1), Empty (2), default (3)} | read-write Implement consistency with mib file definition. |
| powerOper.0 | .1.3.6.1.4.1.89089.1.2.2.2.0 | Power operation | INTEGER { On (1), Off (2), Reset (3)} | read-write Only supports set to reset (3). |

IOCTL

(NMS-IOCTL- MIB. .iso.org.dod.internet.private.enterprises.nms.nmsModule.ioctl.ioctlconfTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| confid.1 | .1.3.6.1.4.1.89089.1.4.3.1.1.1 | node index | Gauge32 (1..255) | read-only Implement consistency with mib file definition. |
| confdirection.1 | .1.3.6.1.4.1.89089.1.4.3.1.2.1 | direction | INTEGER {input(1), output(2)} | read-create Only read operations are supported. |
| conflevel.1 | .1.3.6.1.4.1.89089.1.4.3.1.3.1 | level | INTEGER {low(1), high(2)} | read-create Read and write operations are supported. |
| confdescription.1 | .1.3.6.1.4.1.89089.1.4.3.1.4.1 | describe | OCTET STRING | read-create Read and write operations are supported. |
| confRowStatus.1 | .1.3.6.1.4.1.89089.1.4.3.1.5.1 | RowStatus value | INTEGER {active(1), notReady(3), createandWait(5), destroy(6)} | read-create Only read operations are supported. |

PoE

(NMS-POE- MIB.iso.org.dod.internet.private.enterprises.nms.nmsModule.poe.pseStateTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| pseStateGroupIndex.1 | .1.3.6.1.4.1.89089.1.5.3.1.1.1 | group index | Integer32 | read-only Implement consistency with mib file definition. |
| pseStatePowerConsume.1 | .1.3.6.1.4.1.89089.1.5.3.1.2.1 | Power consumption, the unit is 0.1 watts | Integer32 | read-only Implement consistency with mib file definition. |
| pseStatePoweredPorts.1 | .1.3.6.1.4.1.89089.1.5.3.1.3.1 | The current number of power supply ports | Integer32 | read-only Implement consistency with mib file definition. |
| pseStateReservedPower.1 | .1.3.6.1.4.1.89089.1.5.3.1.4.1 | The percentage of the system to retain power | Integer32 | read-write Implement consistency with mib file definition. |
| pseStateTotalAvailablePower.1 | .1.3.6.1.4.1.89089.1.5.3.1.5.1 | Available power system, the unit of 0.1 watts | Integer32 | read-only Implement consistency with mib file definition. |

(NMS-POE- MIB.iso.org.dod.internet.private.enterprises.nms.nmsModule.poe.portStateTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| portStateGroupIndex.1.1 | .1.3.6.1.4.1.89089.1.5.4.1.1.1.1 | group index | Integer32 | read-only Implement consistency with mib file definition. |
| portStateIndex.1.1 | .1.3.6.1.4.1.89089.1.5.4.1.2.1.1 | port index | Integer32 | read-only Implement consistency with mib file definition. |
| portStateDesc.1.1 | .1.3.6.1.4.1.89089.1.5.4.1.3.1.1 | port description | OCTET STRING | read-only Implement consistency with mib file definition. |
| portStateIcut.1.1 | .1.3.6.1.4.1.89089.1.5.4.1.4.1.1 | The current current of the port, the unit is 0.1mA | Integer32 | read-only Implement consistency with mib file definition. |
| portStatePower.1.1 | .1.3.6.1.4.1.89089.1.5.4.1.5.1.1 | Current power port, the unit of 0.1 watts | Integer32 | read-only Implement consistency with mib file definition. |
| portStateVoltage.1.1 | .1.3.6.1.4.1.89089.1.5.4.1.6.1.1 | The current voltage of the port, the unit is 0.1 volts | Integer32 | read-only Implement consistency with mib file definition. |
| portStateMaximumPower.1.1 | .1.3.6.1.4.1.89089.1.5.4.1.7.1.1 | Port maximum power, the unit for watts | Integer32 | read-write Implement consistency with mib file definition. |
| portStateLegacy.1.1 | .1.3.6.1.4.1.89089.1.5.4.1.8.1.1 | Whether the compatibility check is enabled | INTEGER | read-write Implement consistency with mib file definition. |
| portStatePowerSupplyMode.1.1 | .1.3.6.1.4.1.89089.1.5.4.1.9.1.1 | Port power supply mode | INTEGER | read-only Implement consistency with mib file definition. |

Loop Detect

(NMS-LDET-MIB ldetPortTable!@#.iso.org.dod.internet.private.enterprises.nms.nmsModule.ldet.ldetGlobalModule)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| ldetGlobalEnabled.0 | .1.3.6.1.4.1.89089.1.6.1.1.0 | The enable state of the global loop detection | INTEGER {enabled(1), disabled(2)} | read-write Implement consistency with mib file definition. |
| ldetGlobalInterval.0 | .1.3.6.1.4.1.89089.1.6.1.2.0 | The interval between sending loop detection packets | Integer32 (5..300) | read-write Implement consistency with mib file definition. |

(NMS-LDET-MIB ldetPortTable!@#.iso.org.dod.internet.private.enterprises.nms.nmsModule.ldet.ldetPortTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| ldetPortifIndex.1 | .1.3.6.1.4.1.89089.1.6.4.1.1.1 | Port index | Integer32 | not-accessible Implement consistency with mib file definition. |
| ldetPortEnabled.1 | .1.3.6.1.4.1.89089.1.6.4.1.2.1 | Enable state of port loop detection | INTEGER {enabled(1), disabled(2)} | read-write Implement consistency with mib file definition. |
| ldetPortAction.1 | .1.3.6.1.4.1.89089.1.6.4.1.4.1 | Port processing mode | INTEGER {alarm (1), down(2)} | read-write Implement consistency with mib file definition. |
| ldetPortStatus.1 | .1.3.6.1.4.1.89089.1.6.4.1.5.1 | Port status | INTEGER {normal(0), alarm(1), down(2)} | read-only Implement consistency with mib file definition. |

Power Information

(NMS-SYS-MIB powerTable!@#.iso.org.dod.internet.private.enterprises.nms.nmsModule.sys.powerTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| powerIndex.1 | .1.3.6.1.4.1.89089.1.2.3.1.1.1 | Power index | Gauge32 (1..100) | accessible-for-notify Implement consistency with mib file definition. |
| powerType.1 | .1.3.6.1.4.1.89089.1.2.3.1.2.1 | Power type | INTEGER {unknown(0), ac(1), dc(2)} | read-only Implement consistency with mib file definition. |
| powerState.1 | .1.3.6.1.4.1.89089.1.2.3.1.3.1 | Power state | INTEGER {on(1), off(2)} | read-create Only read operations are supported. |
| powerVoltage.1 | .1.3.6.1.4.1.89089.1.2.3.1.4.1 | Power voltage | Integer32 | read-create Only read operations are supported. |

Cable Detect

(NMS-CABLE-DIAG-

MIB.iso.org.dod.internet.private.enterprises.nms.nmsModule.cableDiag.cableDiagTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| cableDiagPortIndex.1 | .1.3.6.1.4.1.89089.1.9.1.1.1.1 | Index value of interface 1 | Integer (0..65535) | read-only Implement consistency with mib file definition. |
| cableDiagPortType.1 | .1.3.6.1.4.1.89089.1.9.1.1.2.1 | The port type of interface 1 | INTEGER { fastEthernet (0), gigaEthernet (1), other (2)} | read-only Implement consistency with mib file definition. |
| cableDiagLinkStatus.1 | .1.3.6.1.4.1.89089.1.9.1.1.3.1 | Working status of interface 1 | INTEGER { link-down (0), link-up (1), other (2)} | read-only Implement consistency with mib file definition. |
| cableDiagPair1Status.1 | .1.3.6.1.4.1.89089.1.9.1.1.4.1 | Pair 1 status of interface 1 | INTEGER { Ok (0), Open (1), Short (2), open-short (3), crosstalk (4), unknown (5), count (6), no-cable (7), other (8)} | read-only Implement consistency with mib file definition. |
| cableDiagPair2Status.1 | .1.3.6.1.4.1.89089.1.9.1.1.5.1 | Pair 2 status of interface 1 | Same as cableDiagPair1Status | read-only Implement consistency with mib file definition. |
| cableDiagPair3Status.1 | .1.3.6.1.4.1.89089.1.9.1.1.6.1 | Pair 3 status of interface 1 | Same as cableDiagPair1Status | read-only Implement consistency with mib file definition. |
| cableDiagPair4Status.1 | .1.3.6.1.4.1.89089.1.9.1.1.7.1 | Pair 4 status of interface 1 | Same as cableDiagPair1Status | read-only Implement consistency with mib file definition. |
| cableDiagPair1Length.1 | .1.3.6.1.4.1.89089.1.9.1.1.8.1 | Pair 1 length of interface 1 | INTEGER | read-only Implement consistency with mib file definition. |
| cableDiagPair2Length.1 | .1.3.6.1.4.1.89089.1.9.1.1.9.1 | Pair 2 length of interface 1 | INTEGER | read-only Implement consistency with mib file definition. |
| cableDiagPair3Length.1 | .1.3.6.1.4.1.89089.1.9.1.1.10.1 | Pair 3 length of interface 1 | INTEGER | read-only Implement consistency with mib file definition. |
| cableDiagPair4Length.1 | .1.3.6.1.4.1.89089.1.9.1.1.11.1 | Pair 4 length of interface 1 | INTEGER | read-only Implement consistency with mib file definition. |
| cableDiagAction.1 | .1.3.6.1.4.1.89089.1.9.1.1.12.1 | Detection status of interface 1 | INTEGER { Action (1), Processing (2), Other (3)} | read-write Implement consistency with mib file definition. |
| cableDiagStatus.1 | .1.3.6.1.4.1.89089.1.9.1.1.13.1 | Detection result for interface 1 | INTEGER { not-run (1), processing (2), last-test-ok (3), last-test-failed (4)} | read-only Implement consistency with mib file definition. |

Port Control

(NMS-PORT-MIB .iso.org.dod.internet.private.enterprises.nms.nmsModule.pctrl.portControlTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| portControlIndex.1 | .1.3.6.1.4.1.89089.1.7.2.1.1.1 | Index value of interface 1 | Integer32 (1..2147483647) | read-only Implement consistency with mib file definition. |
| portControlAdminStatus.1 | .1.3.6.1.4.1.89089.1.7.2.1.2.1 | Management status of interface 1 | INTEGER { Up (1), Down (2), Testing (3)} | read-write Implement consistency with mib file definition. |
| portControlOperStatus.1 | .1.3.6.1.4.1.89089.1.7.2.1.3.1 | Actual status of interface 1 | INTEGER { Up (1), Down (2), Testing (3), unknown (4), dormant (5), Not Present (6), lowerLayerDown (7) } | read-only Implement consistency with mib file definition. |
| portControlAdminSpeed.1 | .1.3.6.1.4.1.89089.1.7.2.1.4.1 | Set the speed of interface 1 | INTEGER { Auto (0), speed10Mb (1), speed100Mb (2), speed1000Mb (3), unknown (4), speed2500Mb (5), speed10Gb (6), speed25Gb (7), speed40Gb (8), speed100Gb (9)} | read-write Implement consistency with mib file definition. |
| portControlAdminDuplex.1 | .1.3.6.1.4.1.89089.1.7.2.1.5.1 | Set the duplex of interface 1 | INTEGER { Auto (0), Full (1), Half (2), Unknown (3)} | read-write Implement consistency with mib file definition. |
| portControlOperSpeed.1 | .1.3.6.1.4.1.89089.1.7.2.1.6.1 | Actual speed of interface 1 | INTEGER { Other (0), speed10Mb (1), speed100Mb (2), speed1000Mb (3), unknown (4), speed2500Mb (5), speed10Gb (6), speed25Gb (7), speed40Gb (8), speed100Gb (9) } | read-only Implement consistency with mib file definition. |
| portControlOperDuplex.1 | .1.3.6.1.4.1.89089.1.7.2.1.7.1 | Actual duplex of interface 1 | INTEGER { full(1), half(2), unknown(3)} | read-only Implement consistency with mib file definition. |
| portControlNegotiation.1 | .1.3.6.1.4.1.89089.1.7.2.1.8.1 | Auto-negotiation of interface 1 | INTEGER { enabled(1), disabled(2)} | read-write Implement consistency with mib file definition. |
| portControlFlowControl.1 | .1.3.6.1.4.1.89089.1.7.2.1.9.1 | Set the flow control of interface 1 | INTEGER { on(1), off(2), unknown(3)} | read-write Implement consistency with mib file definition. |
| portControlMtu.1 | .1.3.6.1.4.1.89089.1.7.2.1.10.1 | Set the MTU of interface 1 | Integer32 (46..10222) | read-write Implement consistency with mib file definition. |
| portControlComboType.1 | .1.3.6.1.4.1.89089.1.7.2.1.11.1 | Set the combo mode of interface 1 | INTEGER { copper(1), fiber(2), other(3), auto-copper(4), auto-fiber(5)} | read-write Implement consistency with mib file definition. |
| portControlPortMode.1 | .1.3.6.1.4.1.89089.1.7.2.1.12.1 | Actual combo mode of interface 1 | INTEGER { copper(1), fiber(2), other(3)} | read-only Implement consistency with mib file definition. |

MIB file：NMS-IF-EXT-MIB

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| ifMonitorInputRate.1 | .1.3.6.1.4.1.89089.1.10.3.1.2.1 | inbound bandwidth usage | Integer32 | read-only Implement consistency with mib file definition. |
| ifMonitorOutputRate.1 | .1.3.6.1.4.1.89089.1.10.3.1.3.1 | outbound bandwidth usage | Integer32 | read-only Implement consistency with mib file definition. |

Optical Information

(NMS-OPTICAL-

MIB.iso.org.dod.internet.private.enterprises.nms.nmsModule.optical.opticalModuleInfoTable)

| Node name | OIDs | Node description | Value type | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| opticalPortIndex.1 | .1.3.6.1.4.1.89089.1.8.2.1.1.1 | Port index | INTEGER (0..65535) | read-only Implement consistency with mib file definition. |
| opticalTransceiverType.1 | .1.3.6.1.4.1.89089.1.8.2.1.2.1 | Optical module type | OCTET STRING (SIZE (0..32)) | read-only Implement consistency with mib file definition. |
| opticalConnectType.1 | .1.3.6.1.4.1.89089.1.8.2.1.3.1 | Optical module connector type | OCTET STRING (SIZE (0..16)) | read-only Implement consistency with mib file definition. |
| opticalWaveLength.1 | .1.3.6.1.4.1.89089.1.8.2.1.4.1 | Wavelength of the optical module, unit: nm | Integer32 | read-only Implement consistency with mib file definition. |
| opticalVendorName.1 | .1.3.6.1.4.1.89089.1.8.2.1.5.1 | The manufacturer name of the optical module | OCTET STRING (SIZE (0..64)) | read-only Implement consistency with mib file definition. |
| opticalSerialNumber.1 | .1.3.6.1.4.1.89089.1.8.2.1.6.1 | Serial number of the optical module | OCTET STRING (SIZE (0..32)) | read-only Implement consistency with mib file definition. |
| opticalPartNumber.1 | .1.3.6.1.4.1.89089.1.8.2.1.7.1 | Manufacturer's part number | OCTET STRING (SIZE (0..32)) | read-only Implement consistency with mib file definition. |
| opticalTransferDistance.1 | .1.3.6.1.4.1.89089.1.8.2.1.8.1 | Transmission distance of optical module, unit: km | Integer32 | read-only Implement consistency with mib file definition. |
| opticalSupportDDM.1 | .1.3.6.1.4.1.89089.1.8.2.1.9.1 | Whether the optical module supports DDM | INTEGER {support(1), not Support (2)} | read-only Implement consistency with mib file definition. |
| opticalTemperature.1 | .1.3.6.1.4.1.89089.1.8.2.1.10.1 | Optical module temperature, unit: °C | Integer32 | read-only Implement consistency with mib file definition. |
| opticalVoltage.1 | .1.3.6.1.4.1.89089.1.8.2.1.11.1 | Optical module voltage, unit: 0.01V | Integer32 | read-only Implement consistency with mib file definition. |
| opticalBiasCurrent.1 | .1.3.6.1.4.1.89089.1.8.2.1.12.1 | Bias current of optical module, unit: 0.01mA | Integer32 | read-only Implement consistency with mib file definition. |
| opticalRxPower.1 | .1.3.6.1.4.1.89089.1.8.2.1.13.1 | Received power of optical module, unit: 0.01dBm | Integer32 | read-only Implement consistency with mib file definition. |
| opticalTxPower.1 | .1.3.6.1.4.1.89089.1.8.2.1.14.1 | Transmission power of optical module, unit: 0.01dBm | Integer32 | read-only Implement consistency with mib file definition. |
| opticalTempHiAlarm.1 | .1.3.6.1.4.1.89089.1.8.2.1.15.1 | Alarm threshold of high temperature of optical module | Integer32 (-255..255) | read-write Implement consistency with mib file definition. |
| opticalTempHiWarn.1 | .1.3.6.1.4.1.89089.1.8.2.1.16.1 | Warning threshold of high temperature of optical module | Integer32 (-255..255) | read-write Implement consistency with mib file definition. |
| opticalTempLoWarn.1 | .1.3.6.1.4.1.89089.1.8.2.1.17.1 | Warning threshold of low temperature of optical module | Integer32 (-255..255) | read-write Implement consistency with mib file definition. |
| opticalTempLoAlarm.1 | .1.3.6.1.4.1.89089.1.8.2.1.18.1 | Alarm threshold of low temperature of optical module | Integer32 (-255..255) | read-write Implement consistency with mib file definition. |
| opticalVoltHiAlarm.1 | .1.3.6.1.4.1.89089.1.8.2.1.19.1 | Alarm threshold of high voltage of optical module | Integer32 (0..500) | read-write Implement consistency with mib file definition. |
| opticalVoltHiWarn.1 | .1.3.6.1.4.1.89089.1.8.2.1.20.1 | Warning threshold of high voltage of optical module | Integer32 (0..500) | read-write Implement consistency with mib file definition. |
| opticalVoltLoWarn.1 | .1.3.6.1.4.1.89089.1.8.2.1.21.1 | Warning threshold of low voltage of optical module | Integer32 (0..500) | read-write Implement consistency with mib file definition. |
| opticalVoltLoAlarm.1 | .1.3.6.1.4.1.89089.1.8.2.1.22.1 | Alarm threshold of low voltage of optical module | Integer32 (0..500) | read-write Implement consistency with mib file definition. |
| opticalBiasHiAlarm.1 | .1.3.6.1.4.1.89089.1.8.2.1.23.1 | Alarm threshold of high current of optical module | Integer32 (0..50000) | read-write Implement consistency with mib file definition. |
| opticalBiasHiWarn.1 | .1.3.6.1.4.1.89089.1.8.2.1.24.1 | Warning threshold of high current of optical module | Integer32 (0..50000) | read-write Implement consistency with mib file definition. |
| opticalBiasLoWarn.1 | .1.3.6.1.4.1.89089.1.8.2.1.25.1 | Warning threshold of low current of optical module | Integer32 (0..50000) | read-write Implement consistency with mib file definition. |
| opticalBiasLoAlarm.1 | .1.3.6.1.4.1.89089.1.8.2.1.26.1 | Alarm threshold of low current of optical module | Integer32 (0..50000) | read-write Implement consistency with mib file definition. |
| opticalRxHiAlarm.1 | .1.3.6.1.4.1.89089.1.8.2.1.27.1 | Alarm threshold of high received power of optical module | Integer32 (-4000..1000) | read-write Implement consistency with mib file definition. |
| opticalRxHiWarn.1 | .1.3.6.1.4.1.89089.1.8.2.1.28.1 | Warning threshold of high received power of optical module | Integer32 (-4000..1000) | read-write Implement consistency with mib file definition. |
| opticalRxLoWarn.1 | .1.3.6.1.4.1.89089.1.8.2.1.29.1 | Warning threshold of low received power of optical module | Integer32 (-4000..1000) | read-write Implement consistency with mib file definition. |
| opticalRxLoAlarm.1 | .1.3.6.1.4.1.89089.1.8.2.1.30.1 | Alarm threshold of low received power of optical module | Integer32 (-4000..1000) | read-write Implement consistency with mib file definition. |
| opticalTxHiAlarm.1 | .1.3.6.1.4.1.89089.1.8.2.1.31.1 | Alarm threshold of high transmission power of optical module | Integer32 (-4000..1000) | read-write Implement consistency with mib file definition. |
| opticalTxHiWarn.1 | .1.3.6.1.4.1.89089.1.8.2.1.32.1 | Warning threshold of high transmission power of optical module | Integer32 (-4000..1000) | read-write Implement consistency with mib file definition. |
| opticalTxLoWarn.1 | .1.3.6.1.4.1.89089.1.8.2.1.33.1 | Warning threshold of low transmission power of optical module | Integer32 (-4000..1000) | read-write Implement consistency with mib file definition. |
| opticalTxLoAlarm.1 | .1.3.6.1.4.1.89089.1.8.2.1.34.1 | Alarm threshold of low transmission power of optical module | Integer32 (-4000..1000) | read-write Implement consistency with mib file definition. |

Trap Notice

| Node name | OIDs | Node description | field | Maximum access rights and implementation specifications |
| --- | --- | --- | --- | --- |
| coldStart | .1.3.6.1.6.3.1.1.5.1 | System cold start | none |  |
| warmStart | .1.3.6.1.6.3.1.1.5.2 | System warm boot | none |  |
| linkDown | .1.3.6.1.6.3.1.1.5.3 | port down | ifIndex, ifAdminStatus, ifOperStatus |  |
| linkUp | .1.3.6.1.6.3.1.1.5.4 | port up | ifIndex, ifAdminStatus, ifOperStatus |  |
| dot3OamNonThresholdEvent | .1.3.6.1.2.1.158.0.2 | Power down alarm | dot3OamEventLogTimestamp, dot3OamEventLogOui, dot3OamEventLogType, dot3OamEventLogLocation, dot3OamEventLogEventTotal |  |
| pethPsePortOnOffNotification | .1.3.6.1.2.1.105.0.1 | PoE port power supply status changes | pethPsePortDetectionStatus |  |
| powerChange | .1.3.6.1.4.1.89089.1.2.7.1 | Power state changes | powerIndex, powerType, powerState, powerVoltage |  |
| batteryLow | .1.3.6.1.4.1.89089.1.2.7.2 | Battery low | powerIndex |  |
| batteryRecover | .1.3.6.1.4.1.89089.1.2.7.3 | Battery recover | powerIndex |  |
| loopDetect | .1.3.6.1.4.1.89089.1.6.2.1 | Loop detected on a port | ifIndex |  |
| loopRecover | .1.3.6.1.4.1.89089.1.6.2.2 | Loop recovered on a port | ifIndex |  |
| newRoot | .1.3.6.1.2.1.17.0.1 | New root for stp | none |  |
| topologyChange | .1.3.6.1.2.1.17.0.2 | Topology change for stp/rstp | none |  |
| opticalModuleAlarmEvent | .1.3.6.1.4.1.89089.1.8.7.1 | Optical module alarm | opticalAlarmIfIndex, opticalAlarmInfo |  |
| opticalModuleWarnEvent | .1.3.6.1.4.1.89089.1.8.7.2 | Optical module warning | opticalAlarmIfIndex, opticalAlarmInfo |  |